Hardware Connection Procedure

The EV Charging Monitoring System consists of an ESP32 microcontroller, SIM800L GSM module, ZMPT101B voltage sensor, ACS712 current sensor and 16×2 I²C LCD. The following procedure was followed to establish the hardware connections.

Step 1: ESP32 Power Supply

The ESP32 development board is powered through its USB connector or VIN pin. The onboard regulator provides the required 3.3 V supply for the ESP32 operation.

Step 2: SIM800L GSM Module Connection

The SIM800L module is connected to the ESP32 using UART communication.

| ESP32 Pin | SIM800L Pin | Description |
| --- | --- | --- |
| GPIO17 (TX2) | RXD | Transmits AT commands from ESP32 to SIM800L |
| GPIO16 (RX2) | TXD | Receives responses from SIM800L |
| GND | GND | Common Ground Connection |
| External 4.0–4.2 V Supply | VCC | Dedicated power supply for SIM800L |

Note:
The SIM800L requires a dedicated 4.0–4.2 V power supply capable of supplying up to 2 A peak current during GSM transmission. The ESP32 power output should not be used to power the SIM800L directly.

Step 3: Voltage Sensor Connection (ZMPT101B)

The ZMPT101B voltage sensor is used to measure the AC mains voltage.

| ZMPT101B Pin | ESP32 Pin |
| --- | --- |
| OUT | GPIO34 |
| VCC | 5 V |
| GND | GND |

The AC input terminals of the ZMPT101B are connected in parallel with the AC supply whose voltage is to be monitored.

Step 4: Current Sensor Connection (ACS712)

The ACS712 Hall-effect current sensor measures the charging current.

| ACS712 Pin | ESP32 Pin |
| --- | --- |
| OUT | GPIO35 |
| VCC | 5 V |
| GND | GND |

The load current flows through the ACS712 input terminals, while the analog output is connected to the ESP32 ADC pin.

Step 5: LCD Display Connection

A 16×2 LCD with an I²C interface is used for displaying voltage, current, power, energy, and system status.

| LCD Pin | ESP32 Pin |
| --- | --- |
| SDA | GPIO21 |
| SCL | GPIO22 |
| VCC | 5 V |
| GND | GND |

The I²C communication protocol reduces the number of required GPIO pins.

Step 6: Common Ground

All modules share a common ground connection.

The following components are connected to a common GND:

- ESP32

- SIM800L

- ZMPT101B

- ACS712

- LCD

- External power supply

A common ground ensures reliable communication and accurate sensor measurements.

Step 7: AC Input Connections

- The ZMPT101B is connected across the AC mains to measure the supply voltage.

- The ACS712 is connected in series with the EV charging load to measure the load current.

Proper insulation and isolation must be maintained while handling AC mains voltage.

Step 8: Communication Interfaces

| Interface | Connected Devices |
| --- | --- |
| UART | ESP32 ↔ SIM800L |
| I²C | ESP32 ↔ LCD |
| Analog ADC | ESP32 ↔ ZMPT101B, ACS712 |

Final Hardware Connections Summary

| Component | ESP32 GPIO |
| --- | --- |
| ZMPT101B OUT | GPIO34 |
| ACS712 OUT | GPIO35 |
| LCD SDA | GPIO21 |
| LCD SCL | GPIO22 |
| SIM800L RX | GPIO17 (TX2) |
| SIM800L TX | GPIO16 (RX2) |

Important Design Considerations

- Provide a dedicated 4.0–4.2 V, 2 A power supply for the SIM800L module.

- Ensure all modules share a common ground.

- Keep analog sensor wiring short to minimize electrical noise.

- Maintain adequate isolation between the AC mains section and the low-voltage control circuitry.

- Verify all connections before applying power to prevent damage to the components.

This format is suitable for inclusion in an academic report, internship report, or project documentation.

CODE:

#ifndef CONFIG_H

#define CONFIG_H

/******************************************************

* FUNCTION PROTOTYPES

******************************************************/

// LCD

/******************************************************

*            LIBRARIES

******************************************************/

#include <Arduino.h>

#include <math.h>

#include <HardwareSerial.h>

#include <Wire.h>

#include <LiquidCrystal_I2C.h>

void initializeLCD();

void updateLCD();

void lcdError(String message);

void lcdSMS();

void lcdUploadStatus(bool success);

// GSM

bool initializeSIM800();

bool connectGPRS();

void disconnectGPRS();

void restartSIM800();

bool sendATCommand(String command,

String expectedResponse,

unsigned long timeout);

// ThingSpeak

bool uploadThingSpeak();

// SMS

void sendSMS();

// Sensors

float readVoltage();

float readCurrent();

float readRealPower();

bool sensorSelfTest();

void printSensorValues();

// Power

void calculateEnergy();

void calculateStatus();

void resetEnergy();

void printPowerInformation();

/******************************************************

*            PIN DEFINITIONS

******************************************************/

#define VOLTAGE_PIN    34

#define CURRENT_PIN    35

#define SIM800_RX      16

#define SIM800_TX      17

#define LCD_SDA        21

#define LCD_SCL        22

/******************************************************

*            HARDWARE OBJECTS

******************************************************/

HardwareSerial sim800(2);

LiquidCrystal_I2C lcd(0x27,16,2);

/******************************************************

*            THINGSPEAK

******************************************************/

const char WRITE_API_KEY[] = "SIIJI2MM06ZU7NRE";

const char APN[] = "airtelgprs.com";

/******************************************************

*            ADC

******************************************************/

const float ADC_REF = 3.3;

const int ADC_MAX = 4095;

/******************************************************

*            SENSOR CALIBRATION

******************************************************/

const float ZMPT_CALIBRATION = 713.0;

const float ACS712_OFFSET = 2.360;

const float ACS712_SENSITIVITY = 0.066;

/******************************************************

*            ELECTRICITY

******************************************************/

float voltage = 0;

float current = 0;

float power = 0;

float energy = 0;

float electricityCost = 0;

const float UNIT_PRICE = 10.0;

/******************************************************

*            STATUS

******************************************************/

String chargingStatus = "Idle";

String alertStatus = "None";

/******************************************************

*            SMS

******************************************************/

String phoneNumber = "+917339014844";

float nextSMS = 0.01;

/******************************************************

*            TIMERS

******************************************************/

unsigned long previousMillis = 0;

unsigned long lastThingSpeakUpdate = 0;

const unsigned long THINGSPEAK_INTERVAL = 25000;

unsigned long successfulUploads = 0;

unsigned long failedUploads = 0;

/******************************************************

*            FUNCTION PROTOTYPES

******************************************************/

#endif

/******************************************************

*                SETUP

******************************************************/

void setup()

{

Serial.begin(115200);

Serial.println();

Serial.println("====================================");

Serial.println("EV Charging Monitoring System V3.0");

Serial.println("====================================");

//------------- ADC ----------------//

analogReadResolution(12);

analogSetPinAttenuation(VOLTAGE_PIN, ADC_11db);

analogSetPinAttenuation(CURRENT_PIN, ADC_11db);

//------------- LCD ----------------//

initializeLCD();

//------------- GSM ----------------//

//------------- Internet -----------//

if(initializeSIM800())

{

if(connectGPRS())

{

Serial.println("GPRS Connected");

}

else

{

Serial.println("GPRS Connection Failed");

}

}

else

{

lcdError("SIM800 ERROR");

while(true)

{

delay(1000);

}

}

previousMillis = millis();

lastThingSpeakUpdate = millis();

lcd.clear();

lcd.setCursor(0,0);

lcd.print("System Ready");

delay(2000);

lcd.clear();

}

/******************************************************

*                  LOOP

******************************************************/

void loop()

{

//---------------- Read Sensors ----------------//

voltage = readVoltage();

current = readCurrent();

power = readRealPower();

//---------------- Energy ----------------------//

calculateEnergy();

//---------------- Status ----------------------//

calculateStatus();

//---------------- LCD -------------------------//

updateLCD();

//---------------- ThingSpeak ------------------//

if(millis() - lastThingSpeakUpdate >= THINGSPEAK_INTERVAL)

{

lastThingSpeakUpdate = millis();

if(uploadThingSpeak())

{

Serial.println("ThingSpeak Upload Success");

}

else

{

Serial.println("Upload Failed");

disconnectGPRS();

delay(2000);

connectGPRS();

}

}

//---------------- SMS -------------------------//

if(energy >= nextSMS)

{

sendSMS();

nextSMS += 0.01;

}

//---------------- Serial Monitor -------------//

Serial.print("Voltage : ");

Serial.print(voltage,1);

Serial.print(" V");

Serial.print("   Current : ");

Serial.print(current,2);

Serial.print(" A");

Serial.print("   Power : ");

Serial.print(power,1);

Serial.print(" W");

Serial.print("   Energy : ");

Serial.print(energy,3);

Serial.print(" kWh");

Serial.print("   Cost : ");

Serial.print(electricityCost,2);

Serial.print(" Rs");

Serial.print("   Status : ");

Serial.println(chargingStatus);

delay(500);

}

/******************************************************

*                  GSM FUNCTIONS

******************************************************/

bool sendATCommand(String command,

String expectedResponse,

unsigned long timeout)

{

Serial.println("--------------------------------");

Serial.print(">> ");

Serial.println(command);

while(sim800.available())

sim800.read();

sim800.println(command);

String response = "";

unsigned long start = millis();

while(millis()-start < timeout)

{

while(sim800.available())

{

response += (char)sim800.read();

}

if(response.length())

{

Serial.print(response);

}

if(response.indexOf(expectedResponse) != -1)

{

Serial.println("Response OK");

return true;

}

if(response.indexOf("ERROR") != -1 ||

response.indexOf("+CME ERROR") != -1)

{

Serial.println("Response ERROR");

return false;

}

delay(10);

}

Serial.println("Response Timeout");

return false;

}

/******************************************************

*          INITIALIZE SIM800L

******************************************************/

bool initializeSIM800()

{

Serial.println();

Serial.println("Initializing SIM800L...");

sim800.begin(9600, SERIAL_8N1, SIM800_RX, SIM800_TX);

delay(5000);

if(!sendATCommand("AT","OK",5000))

{

Serial.println("SIM800 Not Responding");

return false;

}

sendATCommand("ATE0","OK",3000);

sendATCommand("AT+CMGF=1","OK",3000);

sendATCommand("AT+CSCS=\"GSM\"","OK",3000);

Serial.println("SIM800 Ready");

return true;

}

bool connectGPRS()

{

Serial.println();

Serial.println("=================================");

Serial.println("Connecting GPRS");

Serial.println("=================================");

// Attach GPRS

sendATCommand("AT+SAPBR=0,1","OK",3000);

if(!sendATCommand("AT+CGATT=1","OK",5000))

{

Serial.println("GPRS Attach Failed");

return false;

}

// Close old bearer (ignore response)

sim800.println("AT+SAPBR=0,1");

delay(2000);

while(sim800.available())

{

Serial.write(sim800.read());

}

// Set Connection Type

if(!sendATCommand(

"AT+SAPBR=3,1,\"CONTYPE\",\"GPRS\"",

"OK",

5000))

{

Serial.println("Bearer Type Failed");

return false;

}

// Set APN

String command =

"AT+SAPBR=3,1,\"APN\",\"";

command += APN;

command += "\"";

if(!sendATCommand(command,"OK",5000))

{

Serial.println("APN Failed");

return false;

}

// Open Bearer

sim800.println("AT+SAPBR=1,1");

delay(6000);

while(sim800.available())

{

Serial.write(sim800.read());

}

// Verify Bearer

sim800.println("AT+SAPBR=2,1");

String response="";

unsigned long start=millis();

while(millis()-start<5000)

{

while(sim800.available())

{

char c=sim800.read();

Serial.write(c);

response+=c;

}

if(response.indexOf("+SAPBR: 1,1")!=-1)

{

Serial.println();

Serial.println("GPRS Connected Successfully");

return true;

}

delay(10);

}

Serial.println();

Serial.println("Failed to Open Bearer");

return false;

}

/******************************************************

*          DISCONNECT GPRS

******************************************************/

void disconnectGPRS()

{

Serial.println();

Serial.println("=================================");

Serial.println("Disconnecting GPRS");

Serial.println("=================================");

// Close HTTP Session (Ignore any response)

sim800.println("AT+HTTPTERM");

delay(1000);

while(sim800.available())

{

Serial.write(sim800.read());

}

// Close Bearer (Ignore ERROR if already closed)

sim800.println("AT+SAPBR=0,1");

delay(2000);

while(sim800.available())

{

Serial.write(sim800.read());

}

Serial.println();

Serial.println("GPRS Disconnected");

}

/***************************************************

*          RESTART SIM800

******************************************************/

void restartSIM800()

{

Serial.println();

Serial.println("=================================");

Serial.println("Restarting SIM800L");

Serial.println("=================================");

// Close any active HTTP session

sim800.println("AT+HTTPTERM");

delay(1000);

while(sim800.available())

{

Serial.write(sim800.read());

}

// Close GPRS Bearer

sim800.println("AT+SAPBR=0,1");

delay(3000);

while(sim800.available())

{

Serial.write(sim800.read());

}

Serial.println("Rebooting Module...");

// Do NOT wait for OK

sim800.println("AT+CFUN=1,1");

// Module resets immediately

delay(18000);

Serial.println("Reinitializing...");

initializeSIM800();

if(connectGPRS())

{

Serial.println("SIM800 Restart Successful");

}

else

{

Serial.println("SIM800 Restart Failed");

}

}

/******************************************************

*          THINGSPEAK UPLOAD

******************************************************/

bool uploadThingSpeak()

{

Serial.println();

Serial.println("=================================");

Serial.println("Uploading to ThingSpeak");

Serial.println("=================================");

//---------------- Status ----------------//

int statusCode =

(chargingStatus == "Charging") ? 1 : 0;

//---------------- Alert ----------------//

int alertCode = 0;

if(alertStatus == "Low Voltage")

alertCode = 1;

else if(alertStatus == "High Voltage")

alertCode = 2;

else if(alertStatus == "Over Current")

alertCode = 3;

else if(alertStatus == "Over Load")

alertCode = 4;

//---------------- URL ----------------//

String url =

"http://api.thingspeak.com/update?api_key=";

url += WRITE_API_KEY;

url += "&field1=" + String(voltage,1);

url += "&field2=" + String(current,2);

url += "&field3=" + String(power,1);

url += "&field4=" + String(energy,3);

url += "&field5=" + String(electricityCost,2);

url += "&field6=" + String(statusCode);

url += "&field7=" + String(alertCode);

Serial.println(url);

//---------------- Close Old HTTP ----------------//

sim800.println("AT+HTTPTERM");

delay(1000);

while(sim800.available())

sim800.read();

//---------------- HTTPINIT ----------------//

if(!sendATCommand("AT+HTTPINIT","OK",5000))

return false;

//---------------- CID ----------------//

if(!sendATCommand("AT+HTTPPARA=\"CID\",1","OK",3000))

{

disconnectGPRS();

return false;

}

//---------------- URL ----------------//

String command =

"AT+HTTPPARA=\"URL\",\"";

command += url;

command += "\"";

if(!sendATCommand(command,"OK",5000))

{

disconnectGPRS();

return false;

}

//---------------- HTTPACTION ----------------//

while(sim800.available())

sim800.read();

sim800.println("AT+HTTPACTION=0");

String response = "";

unsigned long start = millis();

while(millis()-start < 25000)

{

while(sim800.available())

{

char c = sim800.read();

Serial.write(c);

response += c;

if(response.indexOf("+HTTPACTION:") != -1)

{

if(response.indexOf(",200,") != -1)

{

goto HTTP_SUCCESS;

}

if(response.indexOf(",601,") != -1)

{

Serial.println();

Serial.println("Network Error");

failedUploads++;

disconnectGPRS();

return false;

}

if(response.indexOf(",603,") != -1)

{

Serial.println();

Serial.println("DNS Error");

failedUploads++;

disconnectGPRS();

return false;

}

if(response.indexOf(",604,") != -1)

{

Serial.println();

Serial.println("Stack Busy");

failedUploads++;

disconnectGPRS();

return false;

}

}

}

}

Serial.println();

Serial.println("HTTP Timeout");

failedUploads++;

disconnectGPRS();

return false;

HTTP_SUCCESS:

Serial.println();

Serial.println("HTTP Success");

//---------------- HTTPREAD ----------------//

while(sim800.available())

sim800.read();

sim800.println("AT+HTTPREAD");

delay(3000);

String body = "";

while(sim800.available())

{

char c = sim800.read();

Serial.write(c);

body += c;

}

body.trim();

//----------------------------------

// Extract Entry ID

//----------------------------------

int bodyStart = body.indexOf("\r\n");

int bodyEnd   = body.lastIndexOf("\r\nOK");

String entryID = "";

if(bodyStart != -1 && bodyEnd != -1)

{

entryID = body.substring(bodyStart + 2, bodyEnd);

entryID.trim();

}

if(entryID == "0")

{

Serial.println();

Serial.println("ThingSpeak Rejected Data");

failedUploads++;

return false;

}

successfulUploads++;

Serial.println();

Serial.println("ThingSpeak Upload Successful");

Serial.print("Entry ID : ");

Serial.println(entryID);

Serial.print("Successful Uploads : ");

Serial.println(successfulUploads);

Serial.print("Failed Uploads : ");

Serial.println(failedUploads);

sim800.println("AT+HTTPTERM");

delay(1000);

while(sim800.available())

{

sim800.read();

}

return true;

}/******************************************************

*              POWER CALCULATIONS

******************************************************/

void calculateEnergy()

{

unsigned long currentMillis = millis();

float elapsedHours =

(currentMillis - previousMillis) / 3600000.0;

previousMillis = currentMillis;

//-----------------------------------------

// Energy Calculation (kWh)

//-----------------------------------------

energy += (power * elapsedHours) / 1000.0;

//-----------------------------------------

// Electricity Cost

//-----------------------------------------

electricityCost = energy * UNIT_PRICE;

}

/******************************************************

*          CHARGING STATUS

******************************************************/

void calculateStatus()

{

//-----------------------------------------

// Default Status

//-----------------------------------------

chargingStatus = "Idle";

alertStatus = "None";

//-----------------------------------------

// Charging Detection

//-----------------------------------------

if(voltage > 180 &&

current > 0.15 &&

power>20)

{

chargingStatus = "Charging";

}

//-----------------------------------------

// Low Voltage

//-----------------------------------------

if(voltage > 0 && voltage < 180)

{

alertStatus = "Low Voltage";

}

//-----------------------------------------

// High Voltage

//-----------------------------------------

else if(voltage > 250)

{

alertStatus = "High Voltage";

}

//-----------------------------------------

// Over Current

//-----------------------------------------

else if(current > 15.0)

{

alertStatus = "Over Current";

}

//-----------------------------------------

// Over Load

//-----------------------------------------

else if(power > 3500)

{

alertStatus = "Over Load";

}

//-----------------------------------------

// Sensor Failure

//-----------------------------------------

if(voltage < 5 && current < 0.05)

{

chargingStatus = "Idle";

alertStatus = "Sensor Check";

}

}

/******************************************************

*          READ AC VOLTAGE

******************************************************/

float readVoltage()

{

const int samples = 2000;

double sum = 0;

//-----------------------------

// Find ADC Offset

//-----------------------------

for(int i=0;i<samples;i++)

{

sum += analogRead(VOLTAGE_PIN);

}

double offset = sum / samples;

//-----------------------------

// RMS Calculation

//-----------------------------

double squareSum = 0;

for(int i=0;i<samples;i++)

{

double adc = analogRead(VOLTAGE_PIN);

double value = adc - offset;

squareSum += value * value;

}

double rmsADC = sqrt(squareSum / samples);

//-----------------------------

// Convert ADC to Voltage

//-----------------------------

double sensorVoltage =

rmsADC *

ADC_REF /

ADC_MAX;

//-----------------------------

// Convert Sensor Voltage

//-----------------------------

double mainsVoltage =

sensorVoltage *

ZMPT_CALIBRATION;

//-----------------------------

// Noise Filter

//-----------------------------

if(mainsVoltage < 5)

mainsVoltage = 0;

return mainsVoltage;

}

/******************************************************

*          READ AC CURRENT

******************************************************/

float readCurrent()

{

const uint16_t samples = 1000;

double offset = 0;

//-------------------------------

// Find Sensor Offset

//-------------------------------

for(uint16_t i = 0; i < samples; i++)

{

offset += analogRead(CURRENT_PIN);

}

offset /= samples;

//-------------------------------

// RMS Calculation

//-------------------------------

double squareSum = 0;

for(uint16_t i = 0; i < samples; i++)

{

double adc = analogRead(CURRENT_PIN);

double value = adc - offset;

squareSum += value * value;

}

double rmsADC = sqrt(squareSum / samples);

//-------------------------------

// ADC -> Voltage

//-------------------------------

double sensorVoltage =

rmsADC * ADC_REF / ADC_MAX;

//-------------------------------

// Voltage -> Current

//-------------------------------

double measuredCurrent =

sensorVoltage / ACS712_SENSITIVITY;

//-------------------------------

// Noise Filter

//-------------------------------

if(measuredCurrent < 0.05)

measuredCurrent = 0;

static float filteredCurrent = 0;

filteredCurrent =

0.85 * filteredCurrent +

0.15 * measuredCurrent;

if(filteredCurrent < 0.05)

filteredCurrent = 0;

return filteredCurrent;

}

/******************************************************

*          REAL POWER

******************************************************/

float readRealPower()

{

const int samples = 1500;

double sumVoltage = 0;

double sumCurrent = 0;

//------------------------------------

// Find ADC Offsets

//------------------------------------

for(int i=0;i<samples;i++)

{

sumVoltage += analogRead(VOLTAGE_PIN);

sumCurrent += analogRead(CURRENT_PIN);

}

double voltageOffset =

sumVoltage /

samples;

double currentOffset =

sumCurrent /

samples;

//------------------------------------

// Instantaneous Power

//------------------------------------

double totalPower = 0;

for(int i=0;i<samples;i++)

{

//---------------- Voltage ----------------//

double rawVoltage =

analogRead(VOLTAGE_PIN);

double instantVoltage =

rawVoltage -

voltageOffset;

instantVoltage =

instantVoltage *

ADC_REF /

ADC_MAX;

instantVoltage *=

ZMPT_CALIBRATION;

//---------------- Current ----------------//

double rawCurrent =

analogRead(CURRENT_PIN);

double instantCurrent =

rawCurrent -

currentOffset;

instantCurrent =

instantCurrent *

ADC_REF /

ADC_MAX;

instantCurrent /=

ACS712_SENSITIVITY;

//---------------- Power ----------------//

totalPower +=

instantVoltage *

instantCurrent;

}

double realPower =

totalPower /

samples;

//------------------------------------

// Remove Negative Power

//------------------------------------

realPower = fabs(realPower);

//------------------------------------

// Noise Filter

//------------------------------------

if(realPower < 5)

realPower = 0;

return realPower;

}

/******************************************************

*          SENSOR SELF TEST

******************************************************/

bool sensorSelfTest()

{

if(readVoltage() < 50)

{

Serial.println("Voltage Sensor Error");

return false;

}

if(readCurrent() > 30)

{

Serial.println("Current Sensor Error");

return false;

}

Serial.println("Sensors OK");

return true;

}

/******************************************************

*      PRINT SENSOR VALUES

******************************************************/

void printSensorValues()

{

Serial.println("----------------------------");

Serial.print("Voltage : ");

Serial.println(voltage);

Serial.print("Current : ");

Serial.println(current);

Serial.print("Power : ");

Serial.println(power);

Serial.println("----------------------------");

}

/******************************************************

*              LCD INITIALIZATION

******************************************************/

void initializeLCD()

{

Wire.begin(LCD_SDA, LCD_SCL);

lcd.init();

lcd.backlight();

lcd.clear();

lcd.setCursor(0,0);

lcd.print("EV CHARGER");

lcd.setCursor(0,1);

lcd.print("Starting...");

delay(2000);

lcd.clear();

}

/******************************************************

*              LCD DISPLAY

******************************************************/

void updateLCD()

{

static byte screen = 0;

static unsigned long previousLCD = 0;

if(millis() - previousLCD < 3000)

return;

previousLCD = millis();

screen++;

if(screen > 2)

screen = 0;

switch(screen)

{

//---------------- SCREEN 1 ----------------//

case 0:

lcd.setCursor(0,0);

lcd.print("V:");

lcd.print("      ");

lcd.setCursor(2,0);

lcd.print(voltage,1);

lcd.setCursor(9,0);

lcd.print("I:");

lcd.print("     ");

lcd.setCursor(11,0);

lcd.print(current,2);

lcd.setCursor(0,1);

lcd.print("P:");

lcd.print("         ");

lcd.setCursor(2,1);

lcd.print(power,1);

lcd.print("W");

break;

//---------------- SCREEN 2 ----------------//

case 1:

lcd.setCursor(0,0);

lcd.print("Energy        ");

lcd.setCursor(0,1);

lcd.print("                ");

lcd.setCursor(0,1);

lcd.print(energy,3);

lcd.print(" kWh");

break;

//---------------- SCREEN 3 ----------------//

case 2:

lcd.setCursor(0,0);

lcd.print("Cost Rs.");

lcd.print("       ");

lcd.setCursor(8,0);

lcd.print(electricityCost,2);

lcd.setCursor(0,1);

if(chargingStatus=="Charging")

{

lcd.print("Charging      ");

}

else

{

lcd.print("Idle          ");

}

break;

}

}

/******************************************************

*          LCD ERROR MESSAGE

******************************************************/

void lcdError(String message)

{

lcd.clear();

lcd.setCursor(0,0);

lcd.print("SYSTEM ERROR");

lcd.setCursor(0,1);

lcd.print(message);

}

/******************************************************

*          UPLOAD STATUS

******************************************************/

void lcdUploadStatus(bool success)

{

lcd.clear();

lcd.setCursor(0,0);

lcd.print("ThingSpeak");

lcd.setCursor(0,1);

if(success)

lcd.print("Upload OK");

else

lcd.print("Upload Fail");

delay(1000);

}

/******************************************************

*          SMS STATUS

******************************************************/

void lcdSMS()

{

lcd.clear();

lcd.setCursor(0,0);

lcd.print("SMS Sending");

lcd.setCursor(0,1);

lcd.print(phoneNumber);

delay(1500);

}

/******************************************************

*              SEND SMS

******************************************************/

void sendSMS()

{

Serial.println();

Serial.println("=================================");

Serial.println("Sending SMS");

Serial.println("=================================");

lcdSMS();

//---------------- Build Message ----------------//

String message;

message += "EV Charging Update\n";

message += "------------------\n";

message += "Voltage : ";

message += String(voltage,1);

message += " V\n";

message += "Current : ";

message += String(current,2);

message += " A\n";

message += "Power : ";

message += String(power,1);

message += " W\n";

message += "Energy : ";

message += String(energy,3);

message += " kWh\n";

message += "Cost : Rs.";

message += String(electricityCost,2);

message += "\n";

message += "Status : ";

message += chargingStatus;

message += "\n";

message += "Alert : ";

message += alertStatus;

//---------------- Clear Buffer ----------------//

while(sim800.available())

sim800.read();

//---------------- Text Mode ----------------//

if(!sendATCommand("AT+CMGF=1","OK",3000))

{

Serial.println("SMS Mode Failed");

return;

}

//---------------- Recipient ----------------//

sim800.print("AT+CMGS=\"");

sim800.print(phoneNumber);

sim800.println("\"");

//---------------- Wait for > Prompt ----------------//

String response = "";

unsigned long start = millis();

while(millis() - start < 5000)

{

while(sim800.available())

{

char c = sim800.read();

Serial.write(c);

response += c;

}

if(response.indexOf(">") != -1)

break;

}

if(response.indexOf(">") == -1)

{

Serial.println("SMS Prompt Timeout");

return;

}

//---------------- Send Message ----------------//

sim800.print(message);

sim800.write(26);

//---------------- Wait for Result ----------------//

response = "";

start = millis();

while(millis() - start < 20000)

{

while(sim800.available())

{

char c = sim800.read();

Serial.write(c);

response += c;

}

if(response.indexOf("+CMGS:") != -1 &&

response.indexOf("OK") != -1)

{

Serial.println();

Serial.println("SMS Sent Successfully");

return;

}

if(response.indexOf("ERROR") != -1 ||

response.indexOf("+CMS ERROR") != -1)

{

Serial.println();

Serial.println("SMS Sending Failed");

return;

}

delay(10);

}

Serial.println();

Serial.println("SMS Timeout");

}